Q6.9

D=64

H=16

30 KHz

400 MHz / 30 KHz = 13 000 cycles

13 000 cycles / 64 inputs = 203

Steps:

- Figure out what parts of the architecture are getting folded

- How much are they getting folded

Plan: After each step check results and document them

- Test nieve implementation over the new parameter set (varied clock frequencies)

- Combine biases when they can be and put them hardcoded in the module

- Start by folding each of the equations into just one unit

  - Make a unit that can calculate rt,x, and then ∀x∈h use that same module

- Make all the multiplication use folded/bit seral shift-add multiplication

- Make all multiplication use folded/bit seral addition

Notes:

- Each module needs clk, reset, enable, and ready signals

Torn apart equations:

rt=σWirxt+bir+Whrht-1+bhr
rt,n=σm=1dWir,n,mxt,m+bir,n+m=1hWhr,n,mht-1,m+bhr,n

nt=tanhWinxt+bin+rt⊙Whnht-1+bhn
nt=tanhm=1dWin,n,mxt,m+bin,n+rt,nm=1hWhn,n,mht-1,m+bhn,n

zt=σWizxt+biz+Whzht-1+bhz
zt,n=σm=1dWiz,n,mxt,m+biz,n+m=1hWhz,n,mht-1,m+bhz,n

ht=1-zt⊙nt+zt⊙ht-1
ht,n=1-zt,n*nt,n+zt,n+ht-1,n

// ============================================================================

// Reset Gate Element Module

// ============================================================================

module gru_reset_gate_element #(

parameter int D = 128,

parameter int H = 256,

parameter int DATA_WIDTH = 16,

parameter int FRAC_BITS = 8

) (

input  logic                     clk,

input  logic                     rst_n,

input  logic                     valid_in,

input  logic signed [DATA_WIDTH-1:0] x_t [D-1:0],

input  logic signed [DATA_WIDTH-1:0] h_t_prev [H-1:0],

input  logic signed [DATA_WIDTH-1:0] W_ir_row [D-1:0],

input  logic signed [DATA_WIDTH-1:0] W_hr_row [H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_ir_n,

input  logic signed [DATA_WIDTH-1:0] b_hr_n,

output logic signed [DATA_WIDTH-1:0] r_t_n,

output logic                          valid_out

);

logic signed [2*DATA_WIDTH-1:0] sum_input;

logic signed [2*DATA_WIDTH-1:0] sum_hidden;

logic signed [2*DATA_WIDTH-1:0] mac_result;

logic signed [DATA_WIDTH-1:0]   pre_activation;

int input_mac_count;

int hidden_mac_count;

typedef enum logic [2:0] {

IDLE,

MAC_INPUT,

MAC_HIDDEN,

ADD_BIAS,

SIGMOID,

DONE

} state_t;

state_t state, next_state;

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

state <= IDLE;

sum_input <= '0;

sum_hidden <= '0;

input_mac_count <= 0;

hidden_mac_count <= 0;

r_t_n <= '0;

valid_out <= 1'b0;

end else begin

state <= next_state;

case (state)

IDLE: begin

if (valid_in) begin

sum_input <= '0;

sum_hidden <= '0;

input_mac_count <= 0;

hidden_mac_count <= 0;

valid_out <= 1'b0;

end

end

MAC_INPUT: begin

sum_input <= sum_input + (W_ir_row[input_mac_count] * x_t[input_mac_count]);

input_mac_count <= input_mac_count + 1;

end

MAC_HIDDEN: begin

sum_hidden <= sum_hidden + (W_hr_row[hidden_mac_count] * h_t_prev[hidden_mac_count]);

hidden_mac_count <= hidden_mac_count + 1;

end

ADD_BIAS: begin

mac_result <= (sum_input >>> FRAC_BITS) + (sum_hidden >>> FRAC_BITS) +

{{DATA_WIDTH{b_ir_n[DATA_WIDTH-1]}}, b_ir_n} +

{{DATA_WIDTH{b_hr_n[DATA_WIDTH-1]}}, b_hr_n};

end

SIGMOID: begin

pre_activation <= mac_result[DATA_WIDTH-1:0];

r_t_n <= sigmoid_approx(mac_result[DATA_WIDTH-1:0]);

valid_out <= 1'b1;

end

DONE: begin

valid_out <= 1'b0;

end

endcase

end

end

always_comb begin

next_state = state;

case (state)

IDLE: if (valid_in) next_state = MAC_INPUT;

MAC_INPUT: if (input_mac_count == D-1) next_state = MAC_HIDDEN;

MAC_HIDDEN: if (hidden_mac_count == H-1) next_state = ADD_BIAS;

ADD_BIAS: next_state = SIGMOID;

SIGMOID: next_state = DONE;

DONE: next_state = IDLE;

endcase

end

function automatic logic signed [DATA_WIDTH-1:0] sigmoid_approx(

input logic signed [DATA_WIDTH-1:0] x

);

logic signed [DATA_WIDTH-1:0] result;

logic signed [DATA_WIDTH-1:0] one = (1 << FRAC_BITS);

logic signed [DATA_WIDTH-1:0] half = (1 << (FRAC_BITS-1));

if (x < -(5*half))

result = 0;

else if (x > (5*half))

result = one;

else

result = ((x >>> 2) + (x >>> 3)) + half;

return result;

endfunction

endmodule

// ============================================================================

// Update Gate Element Module (z_t)

// ============================================================================

module gru_update_gate_element #(

parameter int D = 128,

parameter int H = 256,

parameter int DATA_WIDTH = 16,

parameter int FRAC_BITS = 8

) (

input  logic                     clk,

input  logic                     rst_n,

input  logic                     valid_in,

input  logic signed [DATA_WIDTH-1:0] x_t [D-1:0],

input  logic signed [DATA_WIDTH-1:0] h_t_prev [H-1:0],

input  logic signed [DATA_WIDTH-1:0] W_iz_row [D-1:0],

input  logic signed [DATA_WIDTH-1:0] W_hz_row [H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_iz_n,

input  logic signed [DATA_WIDTH-1:0] b_hz_n,

output logic signed [DATA_WIDTH-1:0] z_t_n,

output logic                          valid_out

);

logic signed [2*DATA_WIDTH-1:0] sum_input;

logic signed [2*DATA_WIDTH-1:0] sum_hidden;

logic signed [2*DATA_WIDTH-1:0] mac_result;

int input_mac_count;

int hidden_mac_count;

typedef enum logic [2:0] {

IDLE,

MAC_INPUT,

MAC_HIDDEN,

ADD_BIAS,

SIGMOID,

DONE

} state_t;

state_t state, next_state;

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

state <= IDLE;

sum_input <= '0;

sum_hidden <= '0;

input_mac_count <= 0;

hidden_mac_count <= 0;

z_t_n <= '0;

valid_out <= 1'b0;

end else begin

state <= next_state;

case (state)

IDLE: begin

if (valid_in) begin

sum_input <= '0;

sum_hidden <= '0;

input_mac_count <= 0;

hidden_mac_count <= 0;

valid_out <= 1'b0;

end

end

MAC_INPUT: begin

sum_input <= sum_input + (W_iz_row[input_mac_count] * x_t[input_mac_count]);

input_mac_count <= input_mac_count + 1;

end

MAC_HIDDEN: begin

sum_hidden <= sum_hidden + (W_hz_row[hidden_mac_count] * h_t_prev[hidden_mac_count]);

hidden_mac_count <= hidden_mac_count + 1;

end

ADD_BIAS: begin

mac_result <= (sum_input >>> FRAC_BITS) + (sum_hidden >>> FRAC_BITS) +

{{DATA_WIDTH{b_iz_n[DATA_WIDTH-1]}}, b_iz_n} +

{{DATA_WIDTH{b_hz_n[DATA_WIDTH-1]}}, b_hz_n};

end

SIGMOID: begin

z_t_n <= sigmoid_approx(mac_result[DATA_WIDTH-1:0]);

valid_out <= 1'b1;

end

DONE: begin

valid_out <= 1'b0;

end

endcase

end

end

always_comb begin

next_state = state;

case (state)

IDLE: if (valid_in) next_state = MAC_INPUT;

MAC_INPUT: if (input_mac_count == D-1) next_state = MAC_HIDDEN;

MAC_HIDDEN: if (hidden_mac_count == H-1) next_state = ADD_BIAS;

ADD_BIAS: next_state = SIGMOID;

SIGMOID: next_state = DONE;

DONE: next_state = IDLE;

endcase

end

function automatic logic signed [DATA_WIDTH-1:0] sigmoid_approx(

input logic signed [DATA_WIDTH-1:0] x

);

logic signed [DATA_WIDTH-1:0] result;

logic signed [DATA_WIDTH-1:0] one = (1 << FRAC_BITS);

logic signed [DATA_WIDTH-1:0] half = (1 << (FRAC_BITS-1));

if (x < -(5*half))

result = 0;

else if (x > (5*half))

result = one;

else

result = ((x >>> 2) + (x >>> 3)) + half;

return result;

endfunction

endmodule

// ============================================================================

// New/Candidate Gate Element Module (n_t)

// ============================================================================

module gru_new_gate_element #(

parameter int D = 128,

parameter int H = 256,

parameter int DATA_WIDTH = 16,

parameter int FRAC_BITS = 8

) (

input  logic                     clk,

input  logic                     rst_n,

input  logic                     valid_in,

input  logic signed [DATA_WIDTH-1:0] x_t [D-1:0],

input  logic signed [DATA_WIDTH-1:0] h_t_prev [H-1:0],

input  logic signed [DATA_WIDTH-1:0] r_t_n,      // Reset gate value for this element

input  logic signed [DATA_WIDTH-1:0] W_in_row [D-1:0],

input  logic signed [DATA_WIDTH-1:0] W_hn_row [H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_in_n,

input  logic signed [DATA_WIDTH-1:0] b_hn_n,

output logic signed [DATA_WIDTH-1:0] n_t_n,

output logic                          valid_out

);

logic signed [2*DATA_WIDTH-1:0] sum_input;

logic signed [2*DATA_WIDTH-1:0] sum_hidden;

logic signed [2*DATA_WIDTH-1:0] sum_hidden_gated;

logic signed [2*DATA_WIDTH-1:0] mac_result;

int input_mac_count;

int hidden_mac_count;

typedef enum logic [2:0] {

IDLE,

MAC_INPUT,

MAC_HIDDEN,

GATE_HIDDEN,

ADD_BIAS,

TANH,

DONE

} state_t;

state_t state, next_state;

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

state <= IDLE;

sum_input <= '0;

sum_hidden <= '0;

sum_hidden_gated <= '0;

input_mac_count <= 0;

hidden_mac_count <= 0;

n_t_n <= '0;

valid_out <= 1'b0;

end else begin

state <= next_state;

case (state)

IDLE: begin

if (valid_in) begin

sum_input <= '0;

sum_hidden <= '0;

input_mac_count <= 0;

hidden_mac_count <= 0;

valid_out <= 1'b0;

end

end

MAC_INPUT: begin

sum_input <= sum_input + (W_in_row[input_mac_count] * x_t[input_mac_count]);

input_mac_count <= input_mac_count + 1;

end

MAC_HIDDEN: begin

sum_hidden <= sum_hidden + (W_hn_row[hidden_mac_count] * h_t_prev[hidden_mac_count]);

hidden_mac_count <= hidden_mac_count + 1;

end

GATE_HIDDEN: begin

// Apply reset gate: r_t_n * (sum_hidden + b_hn_n)

sum_hidden_gated <= (r_t_n * ((sum_hidden >>> FRAC_BITS) +

{{DATA_WIDTH{b_hn_n[DATA_WIDTH-1]}}, b_hn_n})) >>> FRAC_BITS;

end

ADD_BIAS: begin

// Add input contribution and input bias

mac_result <= (sum_input >>> FRAC_BITS) +

{{DATA_WIDTH{b_in_n[DATA_WIDTH-1]}}, b_in_n} +

sum_hidden_gated[DATA_WIDTH-1:0];

end

TANH: begin

n_t_n <= tanh_approx(mac_result[DATA_WIDTH-1:0]);

valid_out <= 1'b1;

end

DONE: begin

valid_out <= 1'b0;

end

endcase

end

end

always_comb begin

next_state = state;

case (state)

IDLE: if (valid_in) next_state = MAC_INPUT;

MAC_INPUT: if (input_mac_count == D-1) next_state = MAC_HIDDEN;

MAC_HIDDEN: if (hidden_mac_count == H-1) next_state = GATE_HIDDEN;

GATE_HIDDEN: next_state = ADD_BIAS;

ADD_BIAS: next_state = TANH;

TANH: next_state = DONE;

DONE: next_state = IDLE;

endcase

end

function automatic logic signed [DATA_WIDTH-1:0] tanh_approx(

input logic signed [DATA_WIDTH-1:0] x

);

logic signed [DATA_WIDTH-1:0] result;

logic signed [DATA_WIDTH-1:0] one = (1 << FRAC_BITS);

logic signed [DATA_WIDTH-1:0] neg_one = -(1 << FRAC_BITS);

// Piecewise linear approximation:

// tanh(x) ≈ -1     if x < -2

// tanh(x) ≈ 0.5x   if -2 ≤ x ≤ 2

// tanh(x) ≈ 1      if x > 2

if (x < -(one << 1))

result = neg_one;

else if (x > (one << 1))

result = one;

else

result = x >>> 1; // 0.5x

return result;

endfunction

endmodule

// ============================================================================

// Final Hidden State Element Module (h_t)

// ============================================================================

module gru_hidden_state_element #(

parameter int DATA_WIDTH = 16,

parameter int FRAC_BITS = 8

) (

input  logic                     clk,

input  logic                     rst_n,

input  logic                     valid_in,

input  logic signed [DATA_WIDTH-1:0] z_t_n,      // Update gate value

input  logic signed [DATA_WIDTH-1:0] n_t_n,      // New candidate value

input  logic signed [DATA_WIDTH-1:0] h_t_prev_n, // Previous hidden state

output logic signed [DATA_WIDTH-1:0] h_t_n,

output logic                          valid_out

);

typedef enum logic [1:0] {

IDLE,

COMPUTE,

DONE

} state_t;

state_t state, next_state;

logic signed [DATA_WIDTH-1:0] one;

logic signed [DATA_WIDTH-1:0] one_minus_z;

logic signed [2*DATA_WIDTH-1:0] term1, term2;

assign one = (1 << FRAC_BITS);

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

state <= IDLE;

h_t_n <= '0;

valid_out <= 1'b0;

one_minus_z <= '0;

term1 <= '0;

term2 <= '0;

end else begin

state <= next_state;

case (state)

IDLE: begin

if (valid_in) begin

valid_out <= 1'b0;

end

end

COMPUTE: begin

// h_t_n = (1 - z_t_n) * n_t_n + z_t_n * h_t_prev_n

one_minus_z <= one - z_t_n;

term1 <= (one - z_t_n) * n_t_n;

term2 <= z_t_n * h_t_prev_n;

h_t_n <= ((term1 + term2) >>> FRAC_BITS);

valid_out <= 1'b1;

end

DONE: begin

valid_out <= 1'b0;

end

endcase

end

end

always_comb begin

next_state = state;

case (state)

IDLE: if (valid_in) next_state = COMPUTE;

COMPUTE: next_state = DONE;

DONE: next_state = IDLE;

endcase

end

endmodule

// ============================================================================

// Complete GRU Cell - Sequential Implementation

// ============================================================================

module gru_cell_sequential #(

parameter int D = 128,

parameter int H = 256,

parameter int DATA_WIDTH = 16,

parameter int FRAC_BITS = 8

) (

input  logic                     clk,

input  logic                     rst_n,

input  logic                     start,

// Input vector x_t

input  logic signed [DATA_WIDTH-1:0] x_t [D-1:0],

// Previous hidden state h_(t-1)

input  logic signed [DATA_WIDTH-1:0] h_t_prev [H-1:0],

// Weight matrices for reset gate

input  logic signed [DATA_WIDTH-1:0] W_ir [H-1:0][D-1:0],

input  logic signed [DATA_WIDTH-1:0] W_hr [H-1:0][H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_ir [H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_hr [H-1:0],

// Weight matrices for update gate

input  logic signed [DATA_WIDTH-1:0] W_iz [H-1:0][D-1:0],

input  logic signed [DATA_WIDTH-1:0] W_hz [H-1:0][H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_iz [H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_hz [H-1:0],

// Weight matrices for new gate

input  logic signed [DATA_WIDTH-1:0] W_in [H-1:0][D-1:0],

input  logic signed [DATA_WIDTH-1:0] W_hn [H-1:0][H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_in [H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_hn [H-1:0],

// Output: new hidden state h_t

output logic signed [DATA_WIDTH-1:0] h_t [H-1:0],

output logic                          done

);

// Intermediate gate values

logic signed [DATA_WIDTH-1:0] r_t [H-1:0];

logic signed [DATA_WIDTH-1:0] z_t [H-1:0];

logic signed [DATA_WIDTH-1:0] n_t [H-1:0];

// Control signals

logic valid_in_r, valid_out_r;

logic valid_in_z, valid_out_z;

logic valid_in_n, valid_out_n;

logic valid_in_h, valid_out_h;

int current_element;

logic signed [DATA_WIDTH-1:0] r_t_n_temp;

logic signed [DATA_WIDTH-1:0] z_t_n_temp;

logic signed [DATA_WIDTH-1:0] n_t_n_temp;

logic signed [DATA_WIDTH-1:0] h_t_n_temp;

// State machine

typedef enum logic [3:0] {

IDLE,

COMPUTE_RESET,

WAIT_RESET,

COMPUTE_UPDATE,

WAIT_UPDATE,

COMPUTE_NEW,

WAIT_NEW,

COMPUTE_HIDDEN,

WAIT_HIDDEN,

NEXT_ELEMENT,

FINISHED

} state_t;

state_t state, next_state;

// Gate element instances

gru_reset_gate_element #(.D(D), .H(H), .DATA_WIDTH(DATA_WIDTH), .FRAC_BITS(FRAC_BITS))

reset_gate_inst (

.clk(clk),

.rst_n(rst_n),

.valid_in(valid_in_r),

.x_t(x_t),

.h_t_prev(h_t_prev),

.W_ir_row(W_ir[current_element]),

.W_hr_row(W_hr[current_element]),

.b_ir_n(b_ir[current_element]),

.b_hr_n(b_hr[current_element]),

.r_t_n(r_t_n_temp),

.valid_out(valid_out_r)

);

gru_update_gate_element #(.D(D), .H(H), .DATA_WIDTH(DATA_WIDTH), .FRAC_BITS(FRAC_BITS))

update_gate_inst (

.clk(clk),

.rst_n(rst_n),

.valid_in(valid_in_z),

.x_t(x_t),

.h_t_prev(h_t_prev),

.W_iz_row(W_iz[current_element]),

.W_hz_row(W_hz[current_element]),

.b_iz_n(b_iz[current_element]),

.b_hz_n(b_hz[current_element]),

.z_t_n(z_t_n_temp),

.valid_out(valid_out_z)

);

gru_new_gate_element #(.D(D), .H(H), .DATA_WIDTH(DATA_WIDTH), .FRAC_BITS(FRAC_BITS))

new_gate_inst (

.clk(clk),

.rst_n(rst_n),

.valid_in(valid_in_n),

.x_t(x_t),

.h_t_prev(h_t_prev),

.r_t_n(r_t[current_element]),

.W_in_row(W_in[current_element]),

.W_hn_row(W_hn[current_element]),

.b_in_n(b_in[current_element]),

.b_hn_n(b_hn[current_element]),

.n_t_n(n_t_n_temp),

.valid_out(valid_out_n)

);

gru_hidden_state_element #(.DATA_WIDTH(DATA_WIDTH), .FRAC_BITS(FRAC_BITS))

hidden_state_inst (

.clk(clk),

.rst_n(rst_n),

.valid_in(valid_in_h),

.z_t_n(z_t[current_element]),

.n_t_n(n_t[current_element]),

.h_t_prev_n(h_t_prev[current_element]),

.h_t_n(h_t_n_temp),

.valid_out(valid_out_h)

);

// Control FSM

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

state <= IDLE;

current_element <= 0;

valid_in_r <= 1'b0;

valid_in_z <= 1'b0;

valid_in_n <= 1'b0;

valid_in_h <= 1'b0;

done <= 1'b0;

end else begin

state <= next_state;

case (state)

IDLE: begin

if (start) begin

current_element <= 0;

done <= 1'b0;

end

end

COMPUTE_RESET: begin

valid_in_r <= 1'b1;

end

WAIT_RESET: begin

valid_in_r <= 1'b0;

if (valid_out_r) begin

r_t[current_element] <= r_t_n_temp;

end

end

COMPUTE_UPDATE: begin

valid_in_z <= 1'b1;

end

WAIT_UPDATE: begin

valid_in_z <= 1'b0;

if (valid_out_z) begin

z_t[current_element] <= z_t_n_temp;

end

end

COMPUTE_NEW: begin

valid_in_n <= 1'b1;

end

WAIT_NEW: begin

valid_in_n <= 1'b0;

if (valid_out_n) begin

n_t[current_element] <= n_t_n_temp;

end

end

COMPUTE_HIDDEN: begin

valid_in_h <= 1'b1;

end

WAIT_HIDDEN: begin

valid_in_h <= 1'b0;

if (valid_out_h) begin

h_t[current_element] <= h_t_n_temp;

end

end

NEXT_ELEMENT: begin

if (current_element < H-1) begin

current_element <= current_element + 1;

end

end

FINISHED: begin

done <= 1'b1;

end

endcase

end

end

always_comb begin

next_state = state;

case (state)

IDLE: if (start) next_state = COMPUTE_RESET;

COMPUTE_RESET: next_state = WAIT_RESET;

WAIT_RESET: if (valid_out_r) next_state = COMPUTE_UPDATE;

COMPUTE_UPDATE: next_state = WAIT_UPDATE;

WAIT_UPDATE: if (valid_out_z) next_state = COMPUTE_NEW;

COMPUTE_NEW: next_state = WAIT_NEW;

WAIT_NEW: if (valid_out_n) next_state = COMPUTE_HIDDEN;

COMPUTE_HIDDEN: next_state = WAIT_HIDDEN;

WAIT_HIDDEN: if (valid_out_h) next_state = NEXT_ELEMENT;

NEXT_ELEMENT: begin

if (current_element >= H-1)

next_state = FINISHED;

else

next_state = COMPUTE_RESET;

end

FINISHED: next_state = IDLE;

endcase

end

endmodule

// ============================================================================

// Complete GRU Cell - Parallel Implementation

// ============================================================================

module gru_cell_parallel #(

parameter int D = 128,

parameter int H = 256,

parameter int DATA_WIDTH = 16,

parameter int FRAC_BITS = 8,

parameter int NUM_PARALLEL = 4

) (

input  logic                     clk,

input  logic                     rst_n,

input  logic                     start,

input  logic signed [DATA_WIDTH-1:0] x_t [D-1:0],

input  logic signed [DATA_WIDTH-1:0] h_t_prev [H-1:0],

// Weight matrices

input  logic signed [DATA_WIDTH-1:0] W_ir [H-1:0][D-1:0],

input  logic signed [DATA_WIDTH-1:0] W_hr [H-1:0][H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_ir [H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_hr [H-1:0],

input  logic signed [DATA_WIDTH-1:0] W_iz [H-1:0][D-1:0],

input  logic signed [DATA_WIDTH-1:0] W_hz [H-1:0][H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_iz [H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_hz [H-1:0],

input  logic signed [DATA_WIDTH-1:0] W_in [H-1:0][D-1:0],

input  logic signed [DATA_WIDTH-1:0] W_hn [H-1:0][H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_in [H-1:0],

input  logic signed [DATA_WIDTH-1:0] b_hn [H-1:0],

output logic signed [DATA_WIDTH-1:0] h_t [H-1:0], output logic done );

// Intermediate gate values

logic signed [DATA_WIDTH-1:0] r_t [H-1:0];

logic signed [DATA_WIDTH-1:0] z_t [H-1:0];

logic signed [DATA_WIDTH-1:0] n_t [H-1:0];

// Control signals for parallel units

logic [NUM_PARALLEL-1:0] valid_in_r, valid_out_r;

logic [NUM_PARALLEL-1:0] valid_in_z, valid_out_z;

logic [NUM_PARALLEL-1:0] valid_in_n, valid_out_n;

logic [NUM_PARALLEL-1:0] valid_in_h, valid_out_h;

int element_index [NUM_PARALLEL-1:0];

int next_element_to_schedule;

int elements_completed;

// Temporary outputs from parallel units

logic signed [DATA_WIDTH-1:0] r_t_n_temp [NUM_PARALLEL-1:0];

logic signed [DATA_WIDTH-1:0] z_t_n_temp [NUM_PARALLEL-1:0];

logic signed [DATA_WIDTH-1:0] n_t_n_temp [NUM_PARALLEL-1:0];

logic signed [DATA_WIDTH-1:0] h_t_n_temp [NUM_PARALLEL-1:0];

// State machine

typedef enum logic [2:0] {

IDLE,

COMPUTE_RESET,

WAIT_RESET_DONE,

COMPUTE_UPDATE,

WAIT_UPDATE_DONE,

COMPUTE_NEW,

WAIT_NEW_DONE,

COMPUTE_HIDDEN,

WAIT_HIDDEN_DONE,

FINISHED

} state_t;

state_t state, next_state;

// Instantiate parallel reset gate elements

genvar i;

generate

for (i = 0; i < NUM_PARALLEL; i++) begin : gen_reset_gates

gru_reset_gate_element #(.D(D), .H(H), .DATA_WIDTH(DATA_WIDTH), .FRAC_BITS(FRAC_BITS))

reset_gate_inst (

.clk(clk),

.rst_n(rst_n),

.valid_in(valid_in_r[i]),

.x_t(x_t),

.h_t_prev(h_t_prev),

.W_ir_row(W_ir[element_index[i]]),

.W_hr_row(W_hr[element_index[i]]),

.b_ir_n(b_ir[element_index[i]]),

.b_hr_n(b_hr[element_index[i]]),

.r_t_n(r_t_n_temp[i]),

.valid_out(valid_out_r[i])

);

end

endgenerate

// Instantiate parallel update gate elements

generate

for (i = 0; i < NUM_PARALLEL; i++) begin : gen_update_gates

gru_update_gate_element #(.D(D), .H(H), .DATA_WIDTH(DATA_WIDTH), .FRAC_BITS(FRAC_BITS))

update_gate_inst (

.clk(clk),

.rst_n(rst_n),

.valid_in(valid_in_z[i]),

.x_t(x_t),

.h_t_prev(h_t_prev),

.W_iz_row(W_iz[element_index[i]]),

.W_hz_row(W_hz[element_index[i]]),

.b_iz_n(b_iz[element_index[i]]),

.b_hz_n(b_hz[element_index[i]]),

.z_t_n(z_t_n_temp[i]),

.valid_out(valid_out_z[i])

);

end

endgenerate

// Instantiate parallel new gate elements

generate

for (i = 0; i < NUM_PARALLEL; i++) begin : gen_new_gates

gru_new_gate_element #(.D(D), .H(H), .DATA_WIDTH(DATA_WIDTH), .FRAC_BITS(FRAC_BITS))

new_gate_inst (

.clk(clk),

.rst_n(rst_n),

.valid_in(valid_in_n[i]),

.x_t(x_t),

.h_t_prev(h_t_prev),

.r_t_n(r_t[element_index[i]]),

.W_in_row(W_in[element_index[i]]),

.W_hn_row(W_hn[element_index[i]]),

.b_in_n(b_in[element_index[i]]),

.b_hn_n(b_hn[element_index[i]]),

.n_t_n(n_t_n_temp[i]),

.valid_out(valid_out_n[i])

);

end

endgenerate

// Instantiate parallel hidden state elements

generate

for (i = 0; i < NUM_PARALLEL; i++) begin : gen_hidden_states

gru_hidden_state_element #(.DATA_WIDTH(DATA_WIDTH), .FRAC_BITS(FRAC_BITS))

hidden_state_inst (

.clk(clk),

.rst_n(rst_n),

.valid_in(valid_in_h[i]),

.z_t_n(z_t[element_index[i]]),

.n_t_n(n_t[element_index[i]]),

.h_t_prev_n(h_t_prev[element_index[i]]),

.h_t_n(h_t_n_temp[i]),

.valid_out(valid_out_h[i])

);

end

endgenerate

// Control FSM

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

state <= IDLE;

next_element_to_schedule <= 0;

elements_completed <= 0;

done <= 1'b0;

valid_in_r <= '0;

valid_in_z <= '0;

valid_in_n <= '0;

valid_in_h <= '0;

for (int j = 0; j < NUM_PARALLEL; j++) begin

element_index[j] <= j;

end

end else begin

state <= next_state;

case (state)

IDLE: begin

if (start) begin

next_element_to_schedule <= NUM_PARALLEL;

elements_completed <= 0;

done <= 1'b0;

// Initialize first batch

for (int j = 0; j < NUM_PARALLEL; j++) begin

element_index[j] <= j;

end

end

end

COMPUTE_RESET: begin

valid_in_r <= '1;

elements_completed <= 0;

end

WAIT_RESET_DONE: begin

valid_in_r <= '0;

for (int j = 0; j < NUM_PARALLEL; j++) begin

if (valid_out_r[j]) begin

r_t[element_index[j]] <= r_t_n_temp[j];

elements_completed <= elements_completed + 1;

if (next_element_to_schedule < H) begin

element_index[j] <= next_element_to_schedule;

next_element_to_schedule <= next_element_to_schedule + 1;

valid_in_r[j] <= 1'b1;

end

end

end

end

COMPUTE_UPDATE: begin

valid_in_z <= '1;

elements_completed <= 0;

next_element_to_schedule <= NUM_PARALLEL;

end

WAIT_UPDATE_DONE: begin

valid_in_z <= '0;

for (int j = 0; j < NUM_PARALLEL; j++) begin

if (valid_out_z[j]) begin

z_t[element_index[j]] <= z_t_n_temp[j];

elements_completed <= elements_completed + 1;

if (next_element_to_schedule < H) begin

element_index[j] <= next_element_to_schedule;

next_element_to_schedule <= next_element_to_schedule + 1;

valid_in_z[j] <= 1'b1;

end

end

end

end

COMPUTE_NEW: begin

valid_in_n <= '1;

elements_completed <= 0;

next_element_to_schedule <= NUM_PARALLEL;

end

WAIT_NEW_DONE: begin

valid_in_n <= '0;

for (int j = 0; j < NUM_PARALLEL; j++) begin

if (valid_out_n[j]) begin

n_t[element_index[j]] <= n_t_n_temp[j];

elements_completed <= elements_completed + 1;

if (next_element_to_schedule < H) begin

element_index[j] <= next_element_to_schedule;

next_element_to_schedule <= next_element_to_schedule + 1;

valid_in_n[j] <= 1'b1;

end

end

end

end

COMPUTE_HIDDEN: begin

valid_in_h <= '1;

elements_completed <= 0;

next_element_to_schedule <= NUM_PARALLEL;

end

WAIT_HIDDEN_DONE: begin

valid_in_h <= '0;

for (int j = 0; j < NUM_PARALLEL; j++) begin

if (valid_out_h[j]) begin

h_t[element_index[j]] <= h_t_n_temp[j];

elements_completed <= elements_completed + 1;

if (next_element_to_schedule < H) begin

element_index[j] <= next_element_to_schedule;

next_element_to_schedule <= next_element_to_schedule + 1;

valid_in_h[j] <= 1'b1;

end

end

end

end

FINISHED: begin

done <= 1'b1;

end

endcase

end

end

always_comb begin

next_state = state;

case (state)

IDLE: if (start) next_state = COMPUTE_RESET;

COMPUTE_RESET: next_state = WAIT_RESET_DONE;

WAIT_RESET_DONE: if (elements_completed >= H) next_state = COMPUTE_UPDATE;

COMPUTE_UPDATE: next_state = WAIT_UPDATE_DONE;

WAIT_UPDATE_DONE: if (elements_completed >= H) next_state = COMPUTE_NEW;

COMPUTE_NEW: next_state = WAIT_NEW_DONE;

WAIT_NEW_DONE: if (elements_completed >= H) next_state = COMPUTE_HIDDEN;

COMPUTE_HIDDEN: next_state = WAIT_HIDDEN_DONE;

WAIT_HIDDEN_DONE: if (elements_completed >= H) next_state = FINISHED;

FINISHED: next_state = IDLE;

endcase

end

endmodule