Implementing Gated Recurrent Units on FPGAs

Aleksander Berezowski

ENEL 592: Undergraduate Thesis

Dr. Onen and Dr. Kinney-Lang

Contents

Executive Summary (Not Done)	4

Introduction (Not Done)	4

Review of Literature	4

Introduction	4

Basic Architecture of a Brain-Computer Interface (BCI) System	4

Gated Recurrent Unit (GRU)	5

Field-Programmable Gate Array (FPGA)	7

GRUs: Theory and Functionality	8

GRUs and BCI	8

Gated Recurrent Units (GRUs)	10

Hardware Acceleration of GRUs	11

Current GRU Implementations on FPGAs	11

Optimization Techniques	14

Critical Analysis and Research Gaps	18

Conclusion	19

Methodology	20

Experimental Design	20

Trial Execution Flow	20

Sample Size and Repetition	21

Variables	21

Independent Variables	21

Dependent Variables	22

Controlled Variables	22

Data Analysis Plan	23

Initial Data Preparation	23

Visualization and Analysis	23

Tools	24

Hardware Tools	24

Software Tools	24

Proof of Concept and Preliminary Findings	24

Proof of Concept	24

Preliminary Findings	25

Project Timeline and Milestones	29

Proposed Chapter Headings for the Thesis	30

References	31

# Executive Summary (Not Done)

- An overview of the proposed research, encapsulating the research question, methodology, and expected outcomes.

# Introduction (Not Done)

- Present the research topic, its background, and significance.

This thesis will address the following research question:

"How does input size, hidden state size, word size, and the usage of shift-and-add multiplication affect resource usage, inference time, power consumption, and accuracy of brain-computer interface gated recurrent unit machine learning models deployed on field-programmable gate arrays compared to current hardware and software implementations?"

This research question explicitly defines the independent variables as input size (d), hidden state size (h), integer bit width (INT_WIDTH), fractional bit width (FRAC_WIDTH), and the usage of shift-and-add multiplication. Further, the dependent variables are outlined as resource usage (LUTs, registers, BRAMs, DSPs), inference time (measured through Worst Negative Slack and timing utilization), power consumption (total, dynamic, and static), and accuracy (MAE and RMSE). Finally, it establishes the context to be gated recurrent unit (GRU) based machine learning (ML) models deployed on field programmable gate arrays (FPGAs) for brain-computer interface (BCI) applications.

The primary objective of this research is to systematically characterize how different design parameters affect the performance metrics of hardware-implemented GRUs. By quantifying the trade-offs between competing design goals, such as speed versus power consumption, this work aims to provide actionable guidance for designing optimal FPGA-based GRUs for BCI systems. Different parameter combinations are hypothesized to yield varying trade-offs, and understanding these relationships is crucial for determining the best GRU design for specific application constraints.

The scope of this research is carefully bounded to ensure depth and rigor within a manageable framework:

Within Scope:

- Systematic exploration of the design space defined by input size, hidden state size, word size, and usage of shift-and-add multiplication.

- Comprehensive measurement of hardware metrics (resource utilization, timing, power) and accuracy metrics (MAE, RMSE).

- Comparison with existing FPGA-based GRU implementations from the literature.

- Analysis conducted on the Xilinx Artix-7 FPGA platform using industry-standard development tools.

Outside Scope:

- Real-world testing with human participants or live EEG data acquisition.

- Modification or improvement of the fundamental GRU algorithm itself.

- Exploration of alternative RNN architectures (such as LSTMs or vanilla RNNs).

- Physical hardware deployment or ASIC implementation.

- Comparison with CPU or GPU implementations (though literature review provides context).

- Investigation of training algorithms or online learning approaches.

This focused scope allows for thorough investigation of FPGA-specific optimizations while maintaining clear boundaries that distinguish this work from related but separate research questions in the BCI and ML domains.

# Review of Literature

## Introduction

The objective of this review is to examine the current state of research on deploying Gated Recurrent Units (GRUs) on Field-Programmable Gate Arrays (FPGAs), specifically in the context of Brain-Computer Interface (BCI) applications.

The scope of this review includes background on BCI, GRUs, and FPGAs, why GRUs are used for BCI, how GRUs work, FPGA-specific optimizations that can be applied to algorithms, and a critical analysis of the current research and gaps.

### Basic Architecture of a Brain-Computer Interface (BCI) System

Brain-Computer Interfaces (BCIs) are a machine learning based system that can recognize and classify brainwaves. They tend to follow five consecutive stages: signal acquisition, preprocessing / signal enhancement, feature extraction, feature classification, and device control [1].

Fig. 1. Basic architecture of a BCI system

Signal Acquisition: Captures brain activity and may also perform noise reduction or artifact processing. This is often done via non-invasive electroencephalography (EEG) by putting electrodes on the scalp to gather electrical signals.

Preprocessing / Signal Enhancement: Prepares the signal in a suitable form for further processing to maximize the signal-to-noise ratio. Noise is caused by nonneural sources (e.g., 60-Hz line noise and eye movements) and neural sources (e.g., EEG features other than those used for communication).

Feature Extraction: Extract parts of the signal that contain discriminative information.

Feature Classification: Convert the extracted feature into a predetermined classification. There is diversity in classification algorithms due to the diversity of BCIs’ intended real-world applications, and classification algorithms are specialized to their intended use case [2].

Device Control: Translate the classified signal into a meaningful output, such as driving a wheelchair.

### Gated Recurrent Unit (GRU)

Machine learning can be used to perform feature extraction and classification from BCI signals [2]. Due to BCI signals being time-dependent, a machine learning algorithm that can do temporal modeling is required. Feedforward neural networks (FNNs) can struggle with this due to them treating inputs independently [3]. To deal with this, Jordan et al. developed recurrent neural networks (RNNs), which are designed to do temporal modeling. Unlike Feedforward Neural Networks (FNNs), where each node passes information only to the next layer, RNNs also feed information back into previous nodes, allowing them to capture and retain temporal dependencies [4]. However, simple RNNs suffer from vanishing and exploding gradient problems, making them struggle to learn long-term dependencies [5]. To address this, Hochreiter et al. proposed Long Short-Term Memory (LSTM) units. LSTMs use gates to selectively retain, update, and output information, solving the vanishing gradient problem while still allowing temporal dependencies to be learned [6].

Gated Recurrent Units (GRUs) are a type of hidden unit used in recurrent neural networks, proposed in 2014 by Cho et al. They are similar to LSTM units but are computationally simpler and more efficient. This is because GRU’s only use two gates, a reset gate and an update gate, to control information flow through the hidden state. The reset gate allows the unit to drop any irrelevant information, allowing a compact representation of important previous state information. The update gate controls how much information from the previous hidden state is carried over to the current hidden state. These two states allow GRUs to capture dependencies over multiple time scales [7].

Fig. 2. Diagrams of different sequence models: RNN, LSTM, GRU [8].

Chung et al. demonstrated that GRUs and LSTMs achieve comparable performance on multiple data sets [9]. As shown in Table 1, Chung et al. evaluated hyperbolic tangent (tanh), GRU, and LSTM-based RNNs using four polyphonic music datasets and two Ubisoft speech signal datasets. Based on the inference metric negative log-probability, a metric used to quantify the uncertainty and confidence of predictions to determine how well a model fits the data [10], GRUs achieved the best performance in four datasets and performed comparably to the other models in the remaining two datasets [9].

Table 1
The average negative log-probabilities of the training and tests [9]

|  |  |  | tanh | GRU | LSTM |
| --- | --- | --- | --- | --- | --- |
| Music Datasets | Nottingham | Train | 3.22 | 2.79 | 3.08 |
|  |  | Test | 3.13 | 3.23 | 3.20 |
|  | JSB Chorales | Train | 8.82 | 6.94 | 8.15 |
|  |  | Test | 9.10 | 8.54 | 8.67 |
|  | MuseData | Train | 5.64 | 5.06 | 5.18 |
|  |  | Test | 6.23 | 5.99 | 6.23 |
|  | Piano-midi | Train | 5.64 | 4.93 | 6.49 |
|  |  | Test | 9.03 | 8.82 | 9.03 |
| Ubisoft Datasets | Ubisoft dataset A | Train | 6.29 | 2.31 | 1.44 |
|  |  | Test | 6.44 | 3.59 | 2.70 |
|  | Ubisoft dataset B | Train | 7.61 | 0.38 | 0.80 |
|  |  | Test | 7.62 | 0.88 | 1.26 |

Despite similar performance, GRUs have a smaller design architecture and fewer computations, shown in Fig. 2. This allows them to be easier to implement in hardware than an LSTM. However, the first hardware implementation of a GRU was not presented until 2021 by Zaghloul et al. [11]

Table 2
Resource Utilization on GRU Hardware Implementation [11]

| Logic Component | GRU | LSTM |
| --- | --- | --- |
| # BUFs | 3 | 6 |
| # DSPs | 26 | 45 |
| # Block RAM | 14 | 23 |
| Memory LUTs | 342 | 496 |
| FF | 10 241 | 16 025 |
| # Slice Registers | 8 507 | 15 951 |
| # Slice LUTs | 5 926 | 9 158 |

As shown in Table 2, Zaghloul et al.’s hardware implementation of a GRU compared to an LSTM used 50% less Buffers (BUFs), 42% less Digital Signal Processors (DSPs), 39% less Block Random Access Memory (Block RAM), 31% less Memory Look-Up Tables (Memory LUTs), 36% less Flip-Flops (FFs), 47% less Slice Registers, and 35% less Slice Look-Up Tables (Slice LUTs).

Table 3
Comparison Implementation Using Altera Arria 10GX FPGA [11]

| Metric | GRU | LSTM |
| --- | --- | --- |
| Performance per Watt | 157.39 | 146.62 |
| Execution Time | 0.879 | 0.927 |

Despite GRUs having a smaller hardware footprint than LSTMs, Zaghloul et al. also found that the GRU had a higher inference performance per watt and lower execution time, as shown in Table 3.

### Field-Programmable Gate Array (FPGA)

BCI systems require a computational unit to execute signal preprocessing, feature extraction, and classification in real time. Since brain signals are noisy, low-amplitude, and time-dependent, this unit must handle both numerical intensity and strict latency constraints. The most commonly considered platforms, Central Processing Units (CPUs), Graphic Processing Unit (GPUs), and Field-Programmable Gate Arrays (FPGAs) are summarized in Table 4.

Table 4
Pros and Cons of Common BCI Platforms [12-14]

| Platform | Pros | Cons |
| --- | --- | --- |
| CPU | Flexible, can handle diverse computing tasks. Widely available and easy to program. Mature software ecosystem. | High power consumption, critical for embedded systems. Not fully optimized for ML operations (matrix additions/multiplications). Lower parallelism. |
| GPU | Optimized for matrix operations. High parallelism. Tools like CUDA enable hardware-specific acceleration. Well-suited for ML training and inference. | Higher power consumption than CPUs in embedded scenarios/ Memory constraints may require model optimization. Bulkier and less portable than low-power solutions. |
| FPGA | Reconfigurable hardware allows custom accelerators. High parallelism. High performance and power efficiency for specific models. Can be optimized at the model level. | Long development time. High design costs for configuring programmable logic. Requires specialized hardware design knowledge. |

FPGAs are well-suited for BCI systems because their reconfigurability allows for flexible deployment and updating of different BCI machine learning models, while their high performance and low power consumption make them ideal for edge deployment close to users. Although they are more complex to program than CPUs or GPUs, continued research into FPGA-based BCI solutions can help overcome this issue.

An FPGA is a type of integrated circuit that can perform digital logic and algorithms. It contains reprogrammable configurable logic blocks and, unlike Application Specific Integrated Circuits (ASICs) that perform a single function, FPGAs can be reprogrammed to perform different algorithms. FPGAs have several upsides. Their ability to be reprogrammed allows for flexibility in their application. Further, FPGAs can often offer a large performance boost to algorithms compared to software implementations, especially for algorithms with integer or fixed-point data with a high degree of parallelism. Finally, FPGAs have a lower development cost than ASICs while providing similar functionality at a lower speed. However, FPGAs also have downsides. Their flexibility also increases the complexity of programming it, which must be weighed against potential algorithm speed improvements. Due to FPGAs implementing digital logic, they struggle with floating-point intensive applications due to the large amount of logic required for dealing with floating-point arithmetic. FPGAs will also not offer a large amount of speed gain for heavily sequential algorithms. Finally, FPGAs have a high barrier to use properly due to their complexity, resulting in hardware designers needing a significant amount of knowledge and experience in digital circuit design to get the most out of FPGAs [15].

Due to FPGAs being able to process the large amount of data that BCI outputs more efficiently than software-based systems, they seem well positioned to assist BCI systems [16-17]. In 2021, Cai et al. implemented an epilepsy detection algorithm on an FPGA, which improved classification performance and reduced power consumption compared to software implementations of the algorithm. As shown in Table 5, Cai et al. found that FPGA based BCI tended to have a higher accuracy than software-based implementations. This shows how FPGA deployment of algorithms can be more efficient compared to their software counterparts [18].

Table 5
Comparison of BCI Deployments Related to Cai et al.’s Deployment [18]

| Reference | Processer | Accuracy (%) |
| --- | --- | --- |
| Güneysu et al. [19] | CPU | 75 |
| Martišius and Damaševičius [20] | CPU | 80.5 |
| Chen et al. [21] | CPU | 83.3 |
| Shyu et al. [16] | FPGA | 89 |
| Lin et al. [22] | FPGA | 91 |
| Cai et al. [18] | FPGA | 94.4 |

Moreover, FPGAs are being used increasingly in BCI due to their high speed, reliability, and integration [23].

## GRUs: Theory and Functionality

### GRUs and BCI

EEG signals used in BCI are often lengthy, one-dimensional, complicated, and nonlinear time sequence signals. Due to these signal characteristics, RNNs are often used to model this data. However, due to gradient explosion or gradient disappearance that RNNs can be prone to, LSTMs or GRUs are preferred compared to RNNs [24]. Rivas et al. compared GRUs and LSTMs and found their performance is comparable over several evaluation metrics, Root Mean Square Error (RMSE), Mean Square Error (MSE), Mean Average Error (MAE), and Symmetric Mean Absolute Percentage Error (SMAPE), thus both architectures can predict and classify EEG signals to reasonable accuracy. It was found that GRUs are more computationally efficient compared to LSTMs due to their smaller architecture. However, GRUs have a smaller memory capacity compared to LSTMs, leading them to struggle with long-term information, and they can struggle to generalize in some cases [25].

Table 6
Comparison of Different Evaluation Metrics [26-28]

| Evaluation Metric | Definition | Strengths |
| --- | --- | --- |
| MSE | MSE=1ni=1nyi-yi2 | More sensitive to outliers than MAE/SMAPE because it penalizes large errors quadratically. Great when large deviations are especially undesirable. |
| RMSE | RMSE=1ni=1nyi-yi2 | More interpretable than MSE because it is in the same units as the target variable. Same other strengths as MSE. |
| MAE | MAE=1ni=1nyi-yi | More robust to outliers than MSE/RMSE because it penalizes errors linearly. Better reflects median performance than extreme cases. |
| SMAPE | SMAPE=100%ni=1nyi-yiyi+yi*0.5 | Expresses errors as percentages, allowing comparison across datasets with different scales. Same other strengths as MAE. |

Furizal et al. conducted a literature review on LSTMs and GRUs on time series data, specifically temperature forecasting, to determine their performance, summarized in Table 7 [29].

Table 7
LSTMs vs GRUs [29]

|  | LSTM | GRU |
| --- | --- | --- |
| Strengths | Better captures long-term dependencies and complex dynamics | Greater consistency and lower average RMSE, stronger prediction accuracy |
| Limitations | Performance may decline with time or specific datasets | Smaller memory capacity may limit very long-term dependencies |
| Conclusion | Popular and widely used, good performance but less stable across datasets | Often outperforms LSTM in accuracy consistency, more stable error distribution |

### Gated Recurrent Units (GRUs)

The original implementation of GRUs was done by calculating the following four functions for each layer [9]:

rt=σWrxt+Urht-1
ht=tanhWxt+Urt⊙ht-1
zt=σWzxt+Uzht-1
ht=1-zt⊙ht-1+zt⊙ht

Where:

- rt: Reset gate.

- ht: Candidate hidden state.

- zt: Update gate.

- ht: Hidden state.

- ⊙ denotes the Hadamard product.

In modern implementations, GRUs are calculated using the following functions [30-31]:

rt=σWirxt+bir+Whrht-1+bhr
nt=tanhWinxt+bin+rt⊙Whnht-1+bhn
zt=σWizxt+biz+Whzht-1+bhz
ht=1-zt⊙nt+zt⊙ht-1

Where:

- Inputs:

  - d∈N: Input feature dimension.

  - h∈N: Number of hidden units.

  - t∈N: Current timestep.

  - n∈N: Batch size (often n=1in deployment).

  - xt∈Rd: Input vector at timestep t.

- Gates and States:

  - rt∈Rh Reset gate at timestep t.

  - zt∈Rh: Update gate at timestep t.

  - nt∈Rh: Candidate (new) hidden state at timestep t.

  - ht∈Rh: Final hidden state at timestep t.

  - ht-1∈Rh: Previous hidden state at timestep t-1.

- Operations:

  - ⊙ denotes the Hadamard product.

  - σ denotes the sigmoid function.

  - tanh denotes the hyperbolic tangent function.

- Weights and Biases:

  - Wir,Wiz,Win∈Rh×d: Input-to-hidden weight matrices for reset, update, and new gates.

  - Whr,Whz,Whn∈Rh×h: Hidden-to-hidden weight matrices for reset, update, and new gates.

  - bir,biz,bin∈Rh: Input-to-hidden bias vectors.

  - bhr,bhz,bhn∈Rh: Hidden-to-hidden bias vectors.

There are three differences between the original and current implementation. First, the candidate activation function ht is now referred to as nt. This is a semantic change that has no impact on the mathematical differences between the equations. Second, the original equations only include weights, whereas modern implementations include both weights and biases. Adding biases allows the activation function, either σ or tanh, to be shifted left or right, which gives the neural network greater training flexibility [32]. Finally, the order of Hadamard product and multiplication was changed in nt. In the original implementation, the Hadamard product in the new hidden state between rt and ht-1 is performed before multiplication with the weights:

ht=tanhWxt+Urt⊙ht-1

This is in contrast to the PyTorch implementation, which is done after the multiplication:

nt=tanhWinxt+bin+rt⊙Whnht-1+bhn

Ignoring the addition of bias terms, the two candidate activation functions are mathematically equivalent and PyTorch states this is for computational efficiency [31].

## Hardware Acceleration of GRUs

### Current GRU Implementations on FPGAs

GRUs were first implemented in hardware by Zaghoul et al. in 2021 [11] and reimplemented by Rizwan et al. in 2025 [33]. Table 8 shows implementation metrics from the model proposed by Rizwan et al., where N/W size refers to the number of RNN neurons, W refers to the size of the input, U refers to the size of the hidden state, and DSPS refers to the number of DSP Slices.

Table 8
Tabular Form of Resource and Speed Analysis [33]

| N/W Size | GRU Speed (μs) |  |  |  |  |  | Resource Utilization |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | W:3 U:2 | W:6 U:2 | W:3 U:4 | W:6 U:4 | W:3 U:8 | W:6 U:8 |  |
| 4 | 1.45 | 1.05 | 1.44 | 1.04 | - | - | LUT: 17 168 FF: 12 410 DSP: 22 DSPS: 26 |
| 8 | 2.63 | 1.83 | 2.61 | 1.81 | 2.60 | 1.70 | LUT: 28 005 FF: 24 406 DSP: 30 DSPS: 50 |
| 12 | 3.71 | 2.98 | 3.78 | 2.58 | 3.76 | 2.56 | LUT: 43 407 FF: 37 728 DSP: 30 DSPS: - |
| 14 | 4.3 | 3.61 | - | - | - | - | LUT: 45 243 FF: 47 407 DSP: 18 DSPS: - |

The unrolled block architecture for a GRU is shown in Fig. 3. It should be noted that the unrolled block architecture implements the original functions for implementing a GRU and not the most current functions due to biases not being included in Fig. 3.

Fig. 3. Block architecture of unrolled GRU [11].

Based on the unrolled block architecture, the GRU hardware implementation is split into two modules: the gate module and the output module.

The gate module implements the reset gate, update gate, and an activation function. It consists of two Multiply-and-Accumulate (MAC) units that are summed. The summation is passed into a sigmoid activation function or tanh activation function and output. An example of the gate module can is seen in Fig. 4.

Fig. 4. GRU gate hardware implementation [11].

The output module is shown in Fig. 5. It consists of two Hadamard products and a summation, before outputting the result.

Fig. 5. GRU output hardware implementation [11].

The hardware implementation of the GRU architecture is seen in Fig. 5. It consists of three gate modules and one output module.

Fig. 6. Hardware implementation of the GRU architecture [11].

To implement the sigmoid activation function, Piece-Wise linear approximation (PLA) is used. There was no justification for why it was used.

σx=0,  &x≤-50.15625+x*0.03125,  &-5<x≤2.3750.375+x*0.125,  &-2.375<x≤-10.5+x*0.25,  &-1<x≤10.625+x*0.125,  &1<x≤2.3750.84375+x*0.03125,  &2.375<x≤51,  &x>5

Due to tanhx=-1+2σ(2x), the following PLA can be used to estimate tanh:

tanhx=-1,  &x≤-2-1+2σ(2x),  &-2<x≤21,  &x>2

Due to all multiplication constants being powers of two, shift-and-add operations are used instead of traditional multiplication.

### Optimization Techniques

Several different optimization techniques can be applied to the hardware implementation of a GRU. This section will specifically talk about using shift-and-add multiplication, bit serial structures, distributed arithmetic, mixed precision, and better implementation of the hyperbolic tangent and sigmoid functions.

#### Shift-And-Add Multiplication

As already shown, GRUs rely on performing many multiplications between inputs and constant weights. Since multiplication is one of the most expensive hardware operations, optimizing these weight multiplications can significantly improve efficiency. When one of the operands is constant (such as the fixed weights during deployment), generic multipliers can be replaced with shift-and-add units, which reduces hardware cost, speeds up computation, and lowers power consumption [34].

The key idea behind shift-and-add multiplication is that any multiplication can be implemented in hardware as a series of bit-shifts and additions, as long as one of the values is constant, such as a constant weight. For example, multiplying a variable x by the binary constant y=11110 can be expressed as:

y*x=24x+23x+22x+21x

which can be achieved by only using bit shift and add operations. However, for large numbers this can result in using up to n adders, where n is the length of the binary constant. However, y can be represented with fewer non-zero digits:

y=25-21=30

Therefore, multiplication by y requires only two shifts and one subtraction instead of four shifts and three additions. This optimization using both additions and subtractions is called Canonical Signed Digit (CSD) representation. By using 1, 0, and -1 (denoted as 1, 0, 1 respectively) to use trinary representation of numbers instead of binary, the number of nonzero digits can be reduced and multiplications can be done more efficiently.

Conversion from binary to CSD is done programmatically:

- If the binary contains at least one group of two or more 1’s:

  - From LSB to MSB, find a string of 1s followed by a 0 (such as 011)

  - Replace the string with its CSD representation (011 to 101)

Using shift-and-add with CSD representation provides a hardware-efficient way to perform the multiplications required by GRUs at each timestep [35-36]. As shown in Fig. 7, CSD shift-and-add multiplication is not optimal, although it outperforms binary shift-and-add multiplication. This is due to CSD only considering one type of graph topology for shift-and-add combinations. To find the most optimal solution, an exhaustive search is used to consider all possible graph topologies [36]. However, this can be difficult to perform and CSD still offers a substantial improvement over binary representation.

Fig. 7. Average number of add/subtract operations required for multiplying by a constant of different bit widths [36].

To perform fixed-point multiplication with shift-and-add units, given two fixed-point numbers a and b in Qn1.m1 and Qn2.m2 formats, respectively, the product ab will be in Qn.m format where n=n1+n2 and m=m1+m2. To find the product ab, the binary point of a and b are ignored, multiplication is performed, and the binary point is reinserted to the left of the mth bit [37].

Shift-and-add multiplication implemented via CSD can help minimize hardware resource costs by replacing multipliers with bit shifts and additions/subtractions. Due to GRUs relying on a substantial amount of multiplication operations, this hardware optimization could have a sizeable impact on hardware usage and efficiency.

#### Bit Serial Structures

Bit serial structures work by calculating results one bit position at a time, compared to bit parallel structures that compute results by using all bits at once. For example, to add two 32-bit integers in parallel takes 32 adders 1 clock cycle. However, using bit serial methodology 1 adder is instead used 32 times. Bit serial structures trades speed for using fewer hardware resources [38].

BCI systems typically sample in the Hz range [39], while FPGAs operate at speeds in the MHz range. This means that an FPGA can run around a million cycles between each set of new data from a BCI. To take advantage of how much faster an FPGA runs, bit serial structures can be used [40]. While they take longer to compute a result, they take up much less hardware, allowing for smaller hardware deployments. It would help keep the deployment hardware small while ensuring maximum utilization of the hardware.

#### Distributed Arithmetic

Distributed arithmetic is a computational operation that forms the dot product of a pair of vectors in a single step by using precomputing and is used for its high efficiency. Due to distributed arithmetic being bit-serial, it has the disadvantage of being slower than bit-parallel processing; however, as previously discussed, this is not an issue for BCI systems.

Distributed arithmetic works by precomputing specific values to help mechanize equations for faster computation. By using known restraints on functions and the properties of distribution, a small quantity of integers can be precalculated and stored to speed up equations with a much larger number of outputs [41].

#### Mixed Precision

Mixed precision refers to using a variety of quantization for numbers throughout a hardware deployment. The goal is to compress model size by needing less storage and hardware resources while preserving accuracy. This can be done by starting with 8 or 16-bit fixed-point numbers and reducing the number of fractional bits in places that do not degrade model accuracy, such as using 4-bit or 8-bit weights instead of full 16-bits [42].

A Gaussian optimization method has been proposed that causes less than a 1% drop in accuracy while reducing the size of a convolutional neural network by 22.66 to 28.93 times, which could be applied to GRU based networks [43].

#### Alternative Implementation of Sigmoid and Hyperbolic Tangent Functions

Soaryaasa et al. compared implementing the hyperbolic tangent function on an FPGA via PLA, LUTs, Range Addressable LUTs (RALUTs), Exponential Approximation with Power of Two (Power of two), and CORDIC Algorithm Approximation. It compared all methods via speed, hardware usage, and power consumption [44]. Results are shown in Table 9 and Table 10.

Table 9
Resource usage and errors of approximation methods [44]

| Approximation Method | LUT % | LUT RAM % | FF % | IO % | BUFG % | ABS E | MSE | RMSE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PWL-3lines | 0.45 | 0 | 0.01 | 41.90 | 3.13 | 0.1482 | 0.0050 | 0.0709 |
| PWL-5lines | 0.53 | 0 | 0.01 | 68.57 | 3.13 | 0.1057 | 0.0026 | 0.0507 |
| LUT | 0.07 | 0 | 0.01 | 14.76 | 3.13 | 0.0012 | 1.0366e-06 | 0.0010 |
| RALUT | 0.04 | 0 | 0.01 | 15.24 | 3.13 | 0.0278 | 0.0004429 | 0.0210 |
| Power of two ex≅21.5x | 0.16 | 0.01 | 0.02 | 2.75 | 3.13 | 0.0384 | 1.5473e-04 | 0.012439 |
| Power of two ex≅21.4375x | 0.16 | 0.01 | 0.02 | 2.75 | 3.13 | 0.0295 | 7.9969e-05 | 0.008942 |
| CORDIC | 2.20 | 0.02 | 0.42 | 15.24 | 3.13 | 0.0012 | 2.1665e-06 | 0.001471 |

Table 10
Delay and power of approximation methods [44]

| Approximation Method | WNS (ns) | Delay (ns) | Period (ns) | Frequency (MHz) | Max Frequency (MHz) | Total Power Consumption (W) |
| --- | --- | --- | --- | --- | --- | --- |
| PWL-3lines | 5.059 | 4.941 | 10 | 100 | 202.38 | 0.142 |
| PWL-5lines | 4.984 | 5.016 | 10 | 100 | 199 | 0.176 |
| LUT-Flat-50 | 4.537 | 5.463 | 10 | 100 | 183.05 | 0.103 |
| LUT-Flat-10 | 5.238 | 4.762 | 10 | 100 | 210 | 0.108 |
| LUT-Linear-50 | 5.284 | 4.716 | 10 | 100 | 212 | 0.155 |
| LUT-Linear-10 | 5.238 | 4.762 | 10 | 100 | 210 | 0.113 |
| RALUT-Flat-50 | 4.537 | 5.463 | 10 | 100 | 183.5 | 0.103 |
| RALUT-Flat-10 | 5.238 | 4.762 | 10 | 100 | 210 | 0.108 |
| RALUT-Linear-50 | 1.844 | 10.154 | 12 | 83.33 | 98.5 | 0.113 |
| RALUT-Linear-10 | 0.717 | 9.283 | 10 | 100 | 107.7 | 0.112 |
| Power of two ex≅21.5x | 0.225 | 9.775 | 10 | 100 | 102.30 | 0.140 |
| Power of two ex≅21.4375x | 0.165 | 9.835 | 10 | 100 | 101.67 | 0.140 |
| CORDIC | 3.679 | 36.321 | 40 | 25 | 27.532 | 0.117 |

When looking at Table 9, the most important metrics to consider are LUT %, LUTRAM %, FF %, Abs E (analogous to MAE), and RMSE. LUT %, LUTRAM %, and FF % all give information on the hardware usage of the design, where lower is better. Abs E and RMSE give information on accuracy of the implementation, where lower is better.

When looking at Table 10, Max Frequency and Total Power Consumption are the most important metrics to consider. Max Frequency gives information on how fast the implementation can run, where higher is better, and Total Power Consumption gives information on how much power the implementation consumes, where lower is better.

Of the options analyzed, RALUT and LUT implementations have the highest max frequency and the lowest power consumption. CORDIC and LUT implementations have the lowest Abs E and RMSE. Finally, LUT and RALUT implementations have the lowest LUT %, LUTRAM %, and FF %. Thus, a LUT implementation should be used for hyperbolic tangent.

Fig. 8. Comparison of tanh and sigmoid functions

The hyperbolic tangent and sigmoid function are similar because they both have a derivative close to zero for majority of the function except for about 8 units around x=0 and being completely continuous functions. Thus, the method of using LUTs to perform a hyperbolic tangent will also work to perform the sigmoid function.

## Critical Analysis and Research Gaps

This review shows the advantage of implementing GRUs on FPGAs for EEG-based BCI systems. GRUs offer a high amount of computational efficiency, while FPGAs offer reconfigurability and hardware optimizations not available to software implementations of GRUs. However, current hardware implementations of GRUs don’t take full advantage of the potential optimizations that FPGAs offer. Table 11 shows a summary of analyzed hardware optimizations that could improve hardware implemented GRUs.

Table 11
Summary of Analyzed Hardware Optimizations

| Optimization | Summary |
| --- | --- |
| Shift-And-Add Multiplication | Replaces hardware multipliers used for constant multiplication with lower cost shift and addition/subtraction operations. This results in a more efficient hardware implementation. |
| Bit Serial Structures | Replaces parallel arithmetic operations done in one clock cycle with serial arithmetic operations done in multiple clock cycles. Although slower, it takes advantage of the low data rate of EEG signals to optimize hardware utilization. |
| Distributed Arithmetic | Replaces runtime multiplications with precomputed partial sums stored in LUTs. This enables more efficient hardware implementation while using more hardware resources. |
| Mixed Precision | Using variable bit-width representations for numbers enables smaller hardware implementations. It trades a small loss in accuracy for more efficient hardware implementation. |
| Alternative Implementation of Sigmoid and Hyperbolic Tangent Functions | Replaces PLA implementation of sigmoid and hyperbolic tangent functions with LUTs. This yields faster and more power-efficient hardware realizations of these functions. |

More research should be done on the possible hardware optimizations for GRUs, especially considering that each hardware optimization will have a different effect on the power consumption, speed, and utilization of the final hardware realization. For example, while shift-and-add multiplication lowers the size of hardware implementations, also implementing bit serial structures will increase the size of hardware implementations. This would result in an unknown shift in the size of the hardware implementation. Thus, future research should be done to evaluate the impact of multiple hardware optimizations.

Furthermore, it is important to determine if power consumption, speed, or utilization is the most important factor when considering hardware optimizations for BCI systems. Given the relatively low frequency of EEG signals compared to FPGA clock speeds, maximizing computational speed may be less important than minimizing power consumption for edge or wearable deployments.

Finally, there appears to be a lack of research on implementing GRUs on FPGAs, likely due to the relative novelty of GRUs and the complexity of FPGA design and programming. Therefore, this seems to lead to FPGAs being either used for BCI signal acquisition or not at all, bottlenecking research in the field of FPGA deployment of BCI ML models.

## Conclusion

This review explored the implementation of GRUs on FPGAs for BCI applications, with a focus on identifying hardware-specific optimizations that can improve efficiency, speed, and power consumption. GRUs are computationally efficient alternatives to LSTM units, offering similar performance with a simpler architecture. FPGAs provide an ideal platform for BCI GRUs that require low latency and high energy efficiency due to their reconfigurable digital logic and parallelism. Existing hardware implementations of GRUs demonstrate the feasibility of GRU deployment but leave substantial room for hardware-specific optimization.

The reviewed hardware optimization techniques, shift-and-add multiplication, bit-serial structures, distributed arithmetic, mixed precision, and improved function approximations, could each improve GRU performance. Shift-and-add multiplication using CSD representation can significantly reduce the number of required multipliers, a major source of power and area consumption. Bit-serial and distributed arithmetic structures can reduce hardware area. Mixed precision quantization provides a trade-off between accuracy and hardware area. Finally, LUT or RALUT implementations of sigmoid and tanh functions offer increased efficiency compared to traditional PLA or CORDIC approaches.

In conclusion, implementing GRUs on FPGAs represents a promising direction for advancing the efficiency, portability, and real-time performance of BCI systems. By incorporating hardware specific optimizations GRU implementations can achieve better efficiency and lower power consumption compared to current implementations.

# Methodology

## Experimental Design

The experimental system comprises several interconnected Python modules and a TCL script that automate the entire design, implementation, and analysis pipeline:

- Python scripts (generate_gru_sv.py, generate_top_level_sv.py, generate_gru_tb_sv.py) automatically generate SystemVerilog code for the GRU module, top-level wrapper, and testbench based on specified parameters.

- A TCL script (run_vivado.tcl) orchestrates the Xilinx Vivado tool to perform simulation, synthesis, optimization, placement, and routing while generating detailed reports.

- Python scripts (extract_metrics.py, float_to_fixed_point.py) parse Vivado-generated reports and simulation outputs to extract hardware metrics and calculate accuracy measurements.

- The main orchestration script (main.py) coordinates all phases, collects results, calculates Mean Absolute Error (MAE) metrics, and compiles data into a structured format for analysis.

### Trial Execution Flow

Each experimental trial follows a structured sequence to ensure consistency and data integrity:

- Environment Initialization: The system clears previous results and initializes tracking files (status.txt) to monitor progress through the large design space.

- Parameter Configuration: For each combination of INT_WIDTH, FRAC_WIDTH, d, and h, the system sets these values as constants in the code generation scripts.

- RTL Generation:

  - generate_gru_sv.py creates the GRU module implementing the mathematical operations (reset gate, update gate, candidate hidden state, and final hidden state computation) with saturating arithmetic and fixed-point multiplication.

  - generate_top_level_sv.py produces a wrapper module that instantiates the GRU with synthesis preservation directives (keep and dont_touch attributes) to prevent optimization from removing logic.

  - generate_gru_tb_sv.py generates a testbench with 100 test vectors using deterministic mathematical functions (sine waves) for reproducible accuracy testing.

- Vivado Execution: The TCL script (run_vivado.tcl) performs the following operations:

  - Creates a Vivado project targeting the Artix-7 FPGA (xc7a100tcsg324-1).

  - Adds all generated SystemVerilog files with appropriate file type settings.

  - Applies timing constraints (100 MHz clock target from constraints.xdc).

  - Runs behavioral simulation for 500ns to generate output predictions.

  - Executes synthesis with resource and timing optimizations.

  - Performs placement and routing.

  - Generates detailed reports on utilization, timing, and power consumption.

- Data Capture:

  - extract_metrics.py parses multiple report formats using regular expressions to extract hardware metrics (LUTs, registers, BRAMs, DSPs, WNS, power consumption).

  - Simulation outputs are written to text files containing fixed-point binary representations of GRU predictions.

  - float_to_fixed_point.py handles conversion between floating-point and fixed-point representations using two's complement encoding.

- Accuracy Calculation: For each group sharing the same d, h, and INT_WIDTH, the implementation with maximum FRAC_WIDTH serves as the ground truth. MAE is computed between all other configurations and this reference using calculate_mae_metrics(). RMSE will be implemented in the future to give a metric to determine if accuracy degrades with more extreme testcases.

- Error Handling: The system implements comprehensive exception handling to capture failures (resource overflow, routing failures, file access errors) and records the failure mode for later analysis.

This structured flow ensures that each trial is executed identically, carryover effects are eliminated through complete regeneration of all files, and data integrity is maintained through automated validation checks. There will be no changes to this execution flow between the current implementation and the implementation used in the thesis.

### Sample Size and Repetition

The sample size for each experimental condition is determined by the design space dimensions. With initial parameter ranges of 7 values each for h, d, INT_WIDTH, and FRAC_WIDTH, the complete design space comprises 2,401 unique configurations. However, not all configurations can be successfully implemented as designs exceeding FPGA resource capacity fail during placement and routing, providing valuable data on the practical limits of different parameter combinations. In future trials, a design space will be tailored to represent realistic BCI applications.

For accuracy evaluation, each GRU configuration processes 100 test vectors. This sample size provides sufficient data points to calculate stable MAE metrics while maintaining reasonable simulation runtime. The test vectors use deterministic functions as inputs to ensure reproducibility across all trials.

The system is designed to support multiple complete runs, though experiments use a single iteration to fully explore the parameter space before assessing the need for repeated measurements. Variance between runs in the same design space subset does not exist because:

- All designs are synthesized with identical constraints and optimization settings.

- Test vectors are deterministic rather than random.

- Hardware metrics are extracted from static analysis reports rather than runtime measurements.

## Variables

### Independent Variables

- INT_WIDTH: Number of integer bits in fixed-point representation (excluding sign bit). Determines the range of representable values. Lower values reduce hardware cost but may cause overflow in accumulation operations.

- FRAC_WIDTH: Number of fractional bits in fixed-point representation. Determines numerical precision. Lower values reduce hardware multiplier complexity but increase quantization error.

- d: Input feature dimension. Determined by the type of BCI used.

- h: Hidden state dimension. Determined during experimentation during GRU training.

### Dependent Variables

- LUTs: Lookup Tables used for combinational logic. Extracted from utilization_report.txt using regex pattern matching for "CLB LUTs" or "Slice LUTs".

- Registers: Flip-flops used for sequential logic. Extracted from utilization_report.txt using regex for "CLB Registers" or "Slice Registers".

- BRAMs: Block RAM tiles used for memory. Extracted from utilization_report.txt using regex for "Block RAM Tile" or "RAMB" variants.

- DSPs: Digital Signal Processor slices (hardware multipliers). Extracted from utilization_report.txt using regex for "DSPs" or "DSP48E".

- WNS (ns): Worst Negative Slack. Extracted from timing_report.txt.

- Total Power (W): Total FPGA power consumption. Extracted from power_report.txt.

- Dynamic Power (W): Power consumed by switching activity. Extracted from power_report.txt.

- Static Power (W): Leakage power consumption. Extracted from power_report.txt.

- Time Utilization: Fraction of clock period utilized. Calculated as (10ns - WNS) / 10ns where 10ns is the target clock period (100 MHz).

- MAE: Mean Absolute Error of outputs versus ground truth. Calculated using calculate_mae() comparing outputs to highest FRAC_WIDTH configuration with matching d, h, INT_WIDTH.

- RMSE: Root Mean Squared Error of outputs versus ground truth. Will be calculated using calculate_rmse() comparing outputs to highest FRAC_WIDTH configuration with matching d, h, INT_WIDTH.

### Controlled Variables

- Target FPGA: Fixed to Artix-7 xc7a100tcsg324-1. This ensures the LUTs have a consistent size and there is a constant amount of LUTs per slice. These are both factors that can affect the final design according to Yan et al. [45]

- Clock Frequency: Fixed to 100 MHz. Provides a consistent timing target.

- Vivado Version: Fixed to 2024.1. According to Yan et al., the CAD Tool can affect the final design [45].

- Synthesis Settings: Identical optimization flags are used. This ensures fair comparison across designs.

- Test Vectors: Deterministic sine-based functions in testbench. Provides reproducible accuracy measurements without random variation. These will be switched to actual BCI signals for better academic merit.

- Weight Values: Identical pre-initialized weights for all configurations.

- Generation Scripts: Although design inputs into the script are altered, the SystemVerilog generation themselves are constant.

## Data Analysis Plan

The experimental data is exported to a CSV file using pandas during the main.py execution phase. Post-processing and visualization are performed in a Jupyter notebook environment to provide interactive exploration of the multi-dimensional design space.

### Initial Data Preparation

The analysis pipeline begins by loading the CSV output and performing data cleaning operations:

- Removal of constant columns (e.g., controlled variables like FPGA target, clock frequency) to focus on variables that change across trials

- Filtering of failed implementations (LUTs = 0) which indicate designs that exceeded resource constraints or failed placement and routing

This preprocessing ensures that subsequent analyses focus on valid, successfully implemented designs while maintaining awareness of the boundary conditions where implementations fail.

### Visualization and Analysis

Three visualizations have been performed: reduced correlation matrix, descriptive statistics table, and a pairplot matrix. However, in the future more visualizations may be used to better understand the data.

#### Reduced Correlation Matrix

A focused correlation analysis examines relationships between the primary independent variables (d, h, FRAC_WIDTH) and the key dependent variables (LUTs, Registers, DSPs, WNS, Time Utilization, MAE, Total Power, Dynamic Power). As the independent variables are expanded to include INT_WIDTH and the dependant variables are expanded to include RMSE, this correlation matrix will be updated. This visualization:

- Clarifies which parameters most strongly influence each hardware metric.

- Supports identification of the dominant factors in the accuracy-resource trade-off.

- Provides quantitative evidence for claims about parameter sensitivity.

- Facilitates comparison with theoretical predictions from the literature.

#### Descriptive Statistics Table

A descript statistics table provides summary statistics (mean, standard deviation, min/max, quartiles) for all variables. This table helps quickly characterize the data and provides context for interpreting individual data points in subsequent analyses.

#### Pairplot Matrix

The pairplot (pairwise scatter plots between all variables with distribution histograms on the diagonal) enables visual inspection of relationships between all variable pairs. This comprehensive visualization:

- Reveals non-linear relationships that correlation coefficients might underestimate.

- Exposes potential threshold effects where metrics change abruptly.

- Supports hypothesis generation for further targeted analysis.

#### Future Analysis

Analysis for the final thesis will also include the following steps, in addition to the visualizations outlined above:

- Box plots to compare distributions across different pairs of dependant and independent variables.

- Manual regression will be attempted to test if the relationship between independent variables and dependent variables can be modelled without having the simulate an entire design space.

- Hypotheses will be made before simulation and answered after all data is collected. Further, an emphasis will be placed on answering why each hypothesis was correct or incorrect.

## Tools

### Hardware Tools

The hardware needed consists of only a computer capable of running Xilinx Vivado. This is because all simulation is done on a virtual FPGA, thus physical deployment is not required for this study.

### Software Tools

#### Xilinx Vivado

Xilinx is used to lint, synthesize, and implement generated SystemVerilog files. It is an industry-standard FPGA development tool that provides accurate metrics on designs. Further, Xilinx supports the Artix-7 FPGA, the chosen FPGA for this project.

#### Python

Scripts that did not required a specific language were written in Python due to its ability to rapidly develop scripts. Further, Python has an extensive library that were helpful for developing the prototype and will continue to be helpful when modifying the prototype for the thesis.

#### SystemVerilog

All hardware description language was written in SystemVerilog. There was no reason to choose it instead of VHDL. However, SystemVerilog was chosen over Verilog due to SystemVerilog being a more modern, updated version of Verilog.

#### TCL

TCL was used for automated use of Xilinx tools. There was no other option for software language, thus it was the only option.

# Proof of Concept and Preliminary Findings

## Proof of Concept

This proof of concept demonstrates an automated tool to explore the design space of GRUs implemented on FPGAs. The system programmatically generates, synthesizes, implements multiple GRU design given several input parameters and reports extracted metrics.

The configurable parameters are the following:

- INT_WIDTH: Number of integer bits in fixed-point representation

- FRAC_WIDTH: Number of fractional bits in fixed-point representation

- d: Input feature dimension (number of input neurons)

- h: Hidden state dimension (number of hidden units)

Generation of the SystemVerilog scripts used by the tool is done by three Python modules:

- generate_gru_sv.py: Generates the GRU module based on the input parameters.

- generate_top_level_sv.py: Generates a top level file that connects to the GRU. This is done to ensure the GRU ports have connections without needing to connect all of them to external I/O. It also ensures no GRU logic gets optimized away by providing load on the ports and preserving signals with synthesis directives.

- generate_gru_tb.sv: Produces the testbench to assess the accuracy of the GRU module.

For each set of design parameters in the design space, all three generation scripts are ran. Then, run_vivado.tcl is executed. This TCL script:

- Creates a new Vivado project with the Artix-7 FPGA (xc7a100tcsg324-1) as the target device.

- Adds all generated SystemVerilog source files.

- Applies timing constraints from constraints.xdc (100 MHz clock target).

- Runs simulation to generate a file of outputs.

- Executes synthesis, optimization, placement, and routing.

- Generates reports on resource utilization, timing analysis, power consumption.

After the TCL script is ran, extract_metrics.py is executed. It parses the outputs from the simulation to assess accuracy and extracts metrics from the Vivado-generated reports. These are all put into an overall table and stored.

Once all the scripts are ran, accuracy analysis is performed. For a given design subset with the same FRAC_WIDTH, d, and h it assumes the file with the larges INT_WIDTH is the ground truth. It then computes the MAE between every other file in the design subset and the ground truth.

The system operates as a minimum viable product demonstrating core functionality. When running the prototype, there were multiple key findings. First, certain designs are too large to implement on an FPGA board, resulting in implementation failures. This suggests bit serial structures could be very valuable to research. Second, the current accuracy evaluation methodology utilizes the highest-resolution fixed-point implementation as a reference baseline. More thought should be put into if using floating-point calculations as the ground truth would be better or worse. Finally, anecdotal evidence suggests larger designs take multiple hours to generate and analyze. This is problematic when searching a large action space consisting of several large designs. Currently, the prototype is currently single threaded. While running multiple threads would speed up analysis, it wasn’t implemented because analysis speed wasn’t a priority, although it may become one when searching larger action spaces.

## Preliminary Findings

Tables 13 to 16 show results on GRUs evaluated across the following design space:

- INT_WIDTH: 6

- FRAC_WIDTH: 2, 4, 6, 8

- d: 4, 5, 6, 7, 8

- h: 4, 5, 6, 7, 8

Table 13
Experimental Results 0 to 19

Table 14
Experimental Results 20 to 39

Table 15
Experimental Results 40 to 59

Table 16
Experimental Results 60 to 79

Table 17 shows a description of the dataset, both independent and dependant variables. The accuracy of the GRUs within the given design space was predicted to be 8.28% due to frac bit resolution ranging from 2 to 8. However, the mean average error is 9.4%. This suggests that error is accumulating throughout the design due to fixed point numbers.

Table 17
Experimental Results Description

Figure 9 shows a reduced correlation matrix with independent variables on the y-axis and dependant variables on the x-axis. It shows that the larger the hidden state, the more hardware is needed. The correlation matrix also shows that the size of a hidden state has a larger impact than the size of the input dimension. Notably, the MAE is strongly inversely correlated with the amount of fractional bits used. It is interesting that WNS, and by extension Time Utilization, do not have strong correlations to any of the independent variables.

Fig. 9. Reduced Correlation Matrix

# Project Timeline and Milestones

Rerun simulation with bigger action space - December 16th

Run the current simulation that tests the effect of D, H, and word length with a bigger, more complete action space that what was currently ran. This should give more complete results.

Analyze Results - December 18th

Analyze the results from the simulation. This includes making a correlation matrix, analyzing scatter plots, and other methods to give a complete insight on the data.

Write conference paper for The Annual International Conference of the IEEE Engineering in Medicine and Biology Society - January 20th

Combine the methodology and findings so far with the background research done into a paper for EMBC 2026.The due date being towards the end of January should avoid the hectic start of semester while still giving a few days for final edits before the submission deadline.

Submit to The Annual International Conference of the IEEE Engineering in Medicine and Biology Society - January 23rd

According to the EMBC 2026 website, January 23rd is the submission deadline for papers.

Implement Shift-Add Multiplication - February 13th

Add the usage of shift-add multiplication as an input design parameter. Hindsight on how the prototype was made reveals that this will require a very substantial rewrite of the prototype, thus justifying three weeks. However, this rewrite should make implementing other hardware optimizations in the future much easier.

Conduct Simulation - February 20th

Given the long runtime of the simulation, a week has been set aside to run the prototype and all of its simulations.

Analyze Data - March 2nd

Analyze the results from the simulation. This includes making a correlation matrix, analyzing scatter plots, and other methods to give a complete insight on the data. Emphasis will be places on comparing designs with shift-add multiplication compared to those without.

Write Thesis - April 2nd

This step entails writing the entire thesis, as outlined in Proposed Chapter Headings for the Thesis. Due to the scope of this step, a month was allocated.

Present Thesis - April 14th

A final presentation on the thesis, important results, notable challenges, etc. will be done towards the end of the semester. It will be a presentation version of the thesis document.

Find Conference to Submit To - April 14th

Find a conference to submit a reformatted version of the thesis to submit to.

Submit To Conference - April 30th

According to the conference’s submission deadline, modify the thesis and submit it to said conference.

# Proposed Chapter Headings for the Thesis

Chapter 1: Abstract

A concise summary of the research problem, methodology, key findings, and major conclusions. This chapter will provide a quick overview of the study’s purpose and outcomes.

Chapter 2: Introduction

Introduces the research topic, establishes the context and motivation, outlines the problem statement, and defines the study’s objectives and scope. It frames why the research is relevant and what questions it seeks to answer.

Chapter 3: Literature Review

Surveys and synthesizes existing academic work related to the topic. This chapter identifies gaps, trends, and theoretical foundations that inform the research design and justify the study’s contribution.

Chapter 4: Methodology

Describes the research approach, data collection methods, experimental setup, and analytical techniques. It explains how the study was conducted and why particular methods were chosen.

Chapter 5: Results

Presents the findings of the study clearly and objectively, often using tables, figures, or statistical summaries. This chapter focuses on reporting outcomes without interpretation.

Chapter 6: Discussion

Interprets the results, links them back to the research questions and literature, and explains their significance. It provides analytical insights, identifies implications, and acknowledges limitations.

Chapter 7: Future Work

Outlines potential directions for extending or improving the study. This chapter highlights unresolved questions, proposed enhancements, and opportunities for broader application.

Chapter 8: Conclusion

Summarizes the research contributions, reinforces the key findings, and reiterates the study’s importance. It provides a final, cohesive statement that closes the research project.

Chapter 9: References

Lists all academic sources cited throughout the document, formatted according to the IEEE citation style.

# References

- L. F. Nicolas-Alonso and J. Gomez-Gil, “Brain Computer Interfaces, a review,” Sensors, vol. 12, no. 2, pp. 1211–1279, Jan. 2012. doi:10.3390/s120201211

- J. R. Wolpaw et al., “Brain-Computer Interface Technology: A review of the first international meeting,” IEEE Transactions on Rehabilitation Engineering, vol. 8, no. 2, pp. 164–173, Jun. 2000. doi:10.1109/tre.2000.847807

- E. Haselsteiner and G. Pfurtscheller, “Using time-dependent neural networks for EEG classification,” IEEE Transactions on Rehabilitation Engineering, vol. 8, no. 4, pp. 457–463, 2000. doi:10.1109/86.895948

- M. I. Jordan, “Serial order: A parallel distributed processing approach,” Advances in Psychology, pp. 471–495, 1997. doi:10.1016/s0166-4115(97)80111-2

- Y. Bengio, P. Simard, and P. Frasconi, “Learning long-term dependencies with gradient descent is difficult,” IEEE Transactions on Neural Networks, vol. 5, no. 2, pp. 157–166, Mar. 1994. doi:10.1109/72.279181

- S. Hochreiter and J. Schmidhuber, "Long Short-Term Memory," in Neural Computation, vol. 9, no. 8, pp. 1735-1780, 15 Nov. 1997, doi: 10.1162/neco.1997.9.8.1735.

- K. Cho et al., “Learning phrase representations using RNN encoder–decoder for statistical machine translation,” Proceedings of the 2014 Conference on Empirical Methods in Natural Language Processing (EMNLP), 2014. doi:10.3115/v1/d14-1179

- AIML.com, “Sequence models compared: RNNS, lstms, Grus, and Transformers,” AIML.com, https://aiml.com/compare-the-different-sequence-models-rnn-lstm-gru-and-transformers/.

- J. Chung, C. Gulcehre, K. Cho, and Y. Bengio, Empirical Evaluation of Gated Recurrent Neural Networks on Sequence Modeling, Dec. 2014.

- Y. Gal and Z. Ghahramani, “Dropout as a Bayesian Approximation: Representing Model Uncertainty in Deep Learning,” Proceedings of The 33rd International Conference on Machine Learning, vol. 48, pp. 1050–1059, 2016.

- Z. S. Zaghloul and N. Elsayed, "The FPGA Hardware Implementation of the Gated Recurrent Unit Architecture," SoutheastCon 2021, Atlanta, GA, USA, 2021, pp. 1-5, doi: 10.1109/SoutheastCon45413.2021.9401819.

- F. Pacini, T. Pacini, G. Lai, A. M. Zocco, and L. Fanucci, “Design and evaluation of CPU-, GPU-, and FPGA-based deployment of a CNN for motor imagery classification in brain-computer interfaces,” Electronics, vol. 13, no. 9, p. 1646, Apr. 2024. doi:10.3390/electronics13091646

- Avnet Silica, “FPGA vs. GPU vs. CPU – hardware options for AI Applications | Avnet Silica,” FPGA vs. GPU vs. CPU – hardware options for AI applications, https://my.avnet.com/silica/resources/article/fpga-vs-gpu-vs-cpu-hardware-options-for-ai-applications/.

- J. Schneider, “FPGA vs. GPU for deep learning applications,” IBM, https://www.ibm.com/think/topics/fpga-vs-gpu.

- R. Wain et al., An overview of FPGAs and FPGA programming; Initial experiences at Daresbury, Nov. 2006.

- K. -K. Shyu, P. -L. Lee, M. -H. Lee, M. -H. Lin, R. -J. Lai and Y. -J. Chiu, "Development of a Low-Cost FPGA-Based SSVEP BCI Multimedia Control System," in IEEE Transactions on Biomedical Circuits and Systems, vol. 4, no. 2, pp. 125-132, April 2010, doi: 10.1109/TBCAS.2010.2042595.

- K. -K. Shyu et al., "Total Design of an FPGA-Based Brain–Computer Interface Control Hospital Bed Nursing System," in IEEE Transactions on Industrial Electronics, vol. 60, no. 7, pp. 2731-2739, July 2013, doi: 10.1109/TIE.2012.2196897.

- Z. Cai et al., “A high performance heterogeneous hardware architecture for Brain Computer Interface,” Biomedical Engineering Letters, vol. 15, no. 1, pp. 217–227, Nov. 2024. doi:10.1007/s13534-024-00438-4

- A. Güneysu and H. L. Akin, "An SSVEP based BCI to control a humanoid robot by using portable EEG device," 2013 35th Annual International Conference of the IEEE Engineering in Medicine and Biology Society (EMBC), Osaka, Japan, 2013, pp. 6905-6908, doi: 10.1109/EMBC.2013.6611145.

- I. Martišius and R. Damaševičius, “A prototype SSVEP based real time BCI gaming system,” Computational Intelligence and Neuroscience, vol. 2016, pp. 1–15, 2016. doi:10.1155/2016/3861425

- W. Chen, S.-K. Chen, Y.-H. Liu, Y.-J. Chen, and C.-S. Chen, “An electric wheelchair manipulating system using SSVEP-based BCI system,” Biosensors, vol. 12, no. 10, p. 772, Sep. 2022. doi:10.3390/bios12100772

- S. Zuo, Y. Xie, L. Wu and J. Wu, "ApaPRFL: Robust Privacy-Preserving Federated Learning Scheme Against Poisoning Adversaries for Intelligent Devices Using Edge Computing," in IEEE Transactions on Consumer Electronics, vol. 70, no. 1, pp. 725-734, Feb. 2024, doi: 10.1109/TCE.2024.3376561.

- M. Aravind and S. S. Babu, "Embedded implementation of brain computer interface using FPGA," 2016 International Conference on Emerging Technological Trends (ICETT), Kollam, India, 2016, pp. 1-5, doi: 10.1109/ICETT.2016.7873633.

- W. Wang, B. Li, and H. Wang, “A novel end-to-end network based on a bidirectional GRU and a self-attention mechanism for denoising of Electroencephalography signals,” Neuroscience, vol. 505, pp. 10–20, Nov. 2022. doi:10.1016/j.neuroscience.2022.10.006

- F. Rivas, J. E. Sierra-Garcia, and J. M. Camara, “Comparison of LSTM- and GRU-type RNN networks for attention and meditation prediction on raw EEG data from low-cost headsets,” Electronics, vol. 14, no. 4, p. 707, Feb. 2025. doi:10.3390/electronics14040707

- T. Chai and R. R. Draxler, “Root mean square error (RMSE) or mean absolute error (mae)? – arguments against avoiding RMSE in the literature,” Geoscientific Model Development, vol. 7, no. 3, pp. 1247–1250, Jun. 2014. doi:10.5194/gmd-7-1247-2014

- C. Willmott and K. Matsuura, “Advantages of the mean absolute error (mae) over the root mean square error (RMSE) in assessing average model performance,” Climate Research, vol. 30, pp. 79–82, 2005. doi:10.3354/cr030079

- R. J. Hyndman and A. B. Koehler, “Another look at measures of forecast accuracy,” International Journal of Forecasting, vol. 22, no. 4, pp. 679–688, Oct. 2006. doi:10.1016/j.ijforecast.2006.03.001

- F. Furizal et al., “Long short-term memory vs gated recurrent unit: A literature review on the performance of deep learning methods in temperature time series forecasting,” International Journal of Robotics and Control Systems, vol. 4, no. 3, pp. 1506–1526, Sep. 2024. doi:10.31763/ijrcs.v4i3.1546

- A. Zhang, Z. C. Lipton, M. Li, and A. J. Smola, Dive into Deep Learning. Cambridge, England: Cambridge University Press, 2024.

- PyTorch, “GRU,” GRU - PyTorch 2.8 documentation, https://docs.pytorch.org/docs/stable/generated/torch.nn.GRU.html.

- Turing, “What Is the Necessity of Bias in Neural Networks?,” Turing, https://www.turing.com/kb/necessity-of-bias-in-neural-networks.

- K. S. Mohammed Rizwan, S. Nikhil Gowda, S. Yashwanth and K. Anuros Thomas, "Custom Hardware Architecture for FPGA-Based Gated Recurrent Unit," 2025 IEEE 14th International Conference on Communications, Circuits and Systems (ICCCAS), Wuhan, China, 2025, pp. 81-86, doi: 10.1109/ICCCAS65806.2025.11102499.

- R. Garcia and A. Volkova, "Toward the Multiple Constant Multiplication at Minimal Hardware Cost," in IEEE Transactions on Circuits and Systems I: Regular Papers, vol. 70, no. 5, pp. 1976-1988, May 2023, doi: 10.1109/TCSI.2023.3241859.

- K. Z. Pekmestzi and P. Kalivas, “Constant number serial pipeline multipliers,” Journal of VLSI signal processing systems for signal, image and video technology, vol. 26, no. 3, pp. 361–368, Nov. 2000. doi:10.1023/a:1026507617721

- Y. Voronenko and M. Püschel, “Multiplierless multiple constant multiplication,” ACM Transactions on Algorithms, vol. 3, no. 2, p. 11, May 2007. doi:10.1145/1240233.1240234

- S. Arar, “Multiplication examples using the fixed-point representation,” All About Circuits, https://www.allaboutcircuits.com/technical-articles/multiplication-examples-using-the-fixed-point-representation/.

- S. TAKAMAEDA-YAMAZAKI, H. NAKATSUKA, Y. TANAKA, and K. KISE, “Ultrasmall: A Tiny Soft processor architecture with multi-bit serial datapaths for fpgas,” IEICE Transactions on Information and Systems, vol. E98.D, no. 12, pp. 2150–2158, 2015. doi:10.1587/transinf.2015pap0022

- S. Ajami, A. Mahnam, and V. Abootalebi, “Development of a practical high frequency brain–computer interface based on steady-state visual evoked potentials using a single channel of EEG,” Biocybernetics and Biomedical Engineering, vol. 38, no. 1, pp. 106–114, 2018. doi:10.1016/j.bbe.2017.10.004

- T. Thongkham and Y. Jewajinda, "A Low-Cost Bit-Serial Hardware Architecture and FPGA implementation of Spiking Neural Network," 2024 28th International Computer Science and Engineering Conference (ICSEC), Khon Kaen, Thailand, 2024, pp. 1-4, doi: 10.1109/ICSEC62781.2024.10770748.

- S. A. White, "Applications of distributed arithmetic to digital signal processing: a tutorial review," in IEEE ASSP Magazine, vol. 6, no. 3, pp. 4-19, July 1989, doi: 10.1109/53.29648.

- A. Shawahna, S. M. Sait, and A. El-Maleh, “FPGA-based accelerators of deep learning networks for learning and classification: A Review,” IEEE Access, vol. 7, pp. 7823–7859, 2019. doi:10.1109/access.2018.2890150

- D. T. Nguyen, H. Kim and H. -J. Lee, "Layer-Specific Optimization for Mixed Data Flow With Mixed Precision in FPGA Design for CNN-Based Object Detectors," in IEEE Transactions on Circuits and Systems for Video Technology, vol. 31, no. 6, pp. 2450-2464, June 2021, doi: 10.1109/TCSVT.2020.3020569.

- S. Sorayaasa, “Different Implementation Methods of TANH on FPGAs For Neural Networks Application,” thesis, 2022

- A. Yan, R. Cheng, and S. J. Wilton, “On the sensitivity of FPGA architectural conclusions to experimental assumptions, tools, and Techniques,” Proceedings of the 2002 ACM/SIGDA tenth international symposium on Field-programmable gate arrays  - FPGA ’02, 2002. doi:10.1145/503070.503071